ALE Information

ALE INFORMATION FILE...Stitched together and updated by occasional WUN

contributor Jim Dunnett (RGA).

This fourth update to the original file of 31 December 1999 has been

written to support Charles Brain's latest version of the program,

specially modified for WUN-ers, version 1.03k. For which very many

thanks; this version is much more stable and user-friendly than any of

the previous ones.

If anyone finds any disinformation in the following notes, or has any

suggestions for additions or improvements, I would be delighted to hear

from them via:

sideband@btinternet.com

sparta@freeuk.com

(There are no page-breaks in this file)

Contains:

o  An Intro to ALE, by Day Watson.

o  Some ALE references, Internet Sites with

Information etc.

o  Setting up and running the Program.

o  Using the program as a scanner.

Organising scan files.

o  Notes on the ALE Controller Program by

the Author, Charles Brain, G4GUO.

o  Identities of some of the ALE addresses seen so

far, and a list of some ALE frequencies seen up

to mid-January 2000).

o  An Interpretation exercise.

--------------------------------------------------------------------------

[From Day Watson, published in one of the WUN newsletters]

::: Automatic Link Establishment (ALE) - an introduction

-1-

The problem with setting up a circuit on HF is that it can be time

consuming.  Modern technology and computers have now made it possible to

automate this function achieving a higher percentage of first time and

reliable link-up than can be obtained with an operator.

A number of stations associated with a particular organisation each make

use of a single SSB transmitter/receiver (or transceiver) under the

control of an ALE Controller.  This group of stations will operate on a

selection of frequencies thereby coping with the variants of link quality

brought about by both distance and time, and of course interference.

The ALE controller of the master and/or the outstations can periodically

or irregularly send out a "sounding" burst.  There are a number of

variants on this theme but I won't go into the complexity at this time.

The sounding burst may/may not require a response from other stations but

each station can be identified.  When any station is not sending it must

conversely be receiving.  Not only that its ALE controller will be

causing its receiver to scan through the designated frequency channels

specific to that net's operation (this is normally done on the basis of 2

or 5 channels per second) where the controller software is looking for a

data burst containing its unique address or an address for which it is

part of multiple station operations.

Whilst normal operations take place there is a continual process of

link quality analysis (LQA).  The receiving station measures this

quality based on a bit error ratio and a signal/noise ratio (to

describe in its simplest terms).  The result of the analysis is to give a

score for that particular burst, the score being stored in memory.  Over

a period of time the ALE controller knows the current best frequency to

employ when wishing to link with an other.

Station ABC wishes to link with station XYZ.  It's ALE controller uses

XYZ's identity in the call burst having set its transmitter to the

computed best frequency as based on the stored data.  The receiver

at the addressed station is scanning all the frequencies in the set and

stops when a burst appears on one of these channels.  It then determines

that it is being addressed and link-up is established.  There is no

need for the receiving operator to continually watch a set of frequencies

- he/she can be alerted which required.

ALE is reputed to produce 80-90% first time connections as against the

estimated 35% when using traditional/manual procedures.  Use of

scanning eliminates the need for call scheduling and frequency/time

coordination.

The ALE controller in the linkup procedures uses tones (we'll enlarge on

this later) to/from the SSB equipment.  Once the link is established

the controller has the option of passing data in a low speed manner

using its internal provisions.  Alternatively it may be switched

manually or automatically to a second modem which provides high speed

FSK/PSK data, or voice, as far as the information transfer is concerned.

So there in simple terms is what ALE is all about.

-2-

What about the ALE waveform?  It is designed to pass through the AF

passband of standard SSB equipment.  It consists of 8 tones (MFSK). They

are located 250hz apart from 750hz to 2500hz.  The centre of the system

is at 1625 khz.  Whilst some of my colleagues use the assigned frequency

and add the 1.625 khz subcarrier offset I prefer to use the suppressed

carrier value for a number of reasons - eg 18003/carrier against

18004.6/assigned - but that's another story.  However if one tunes to

18003/usb one should eventually

a.  hear a short watery/burbling kind of sound (always difficult to

describe sound in the written word), and

b.  see the tones in the USB if one has an audio spectrum display

starting with the simplest "Spectrogram" as described in this column

a couple of months back.

Each tone is 8mS in duration.  This gives 125 symbols per second.  With

8 tones (symbols or elements) we can support 3 data bits per symbol.

This results in a transmitted data rate of 375 bits per second.

ALE uses a standard word consisting of a 3-bit preamble and three 7-bit

ASCII character fields; a total of 24 bits per word.  The function of

each ALE word is designated by the code in the preamble and is related

to ALE's basic capabilities.  Eight word types are used:

TO     THIS IS     THIS WAS     DATA     REPEAT     THRU     COMMAND

FROM.

Depending on the preamble the 7-bit ASCII character fields are used to

specify individual address characters, or as ASCII text.

Robustness of the system is achieved by the utilisation of a system of

triple redundancy in the transmission of ALE data, by interleaving the

data, and by use of Golay forward error correction (fec).

The ALE system is designated Mil.Std 188-141A and is being taken up by

others outside the US military/SHARES network - Danish Mil., Algerian

diplo, Romanian diplo, Swedish diplo to name but a few.

If you wish to dig deeper into the system I suggest you look at the web

site   http://www.its.bldrdoc.gov/pub/oa-rpt/hf-ale/handbook/

My experience of this site is that the zipped report at the end failed

to unzip in spite of several downloads and I had to use the .pdf files

listed.

-3-

So much for the theory.  It is now possible to decode/watch these

stations linking up etc.  There is a neat program in the form of a

188-141A Ale Controller.  It can be used for both Tx/Rx functions but

for the UTE monitor the receiver side is fine.  It was written for the

personal amusement of the author, but is available on his website for

others with an interest in the subject. Check out:

http://www.chbrain.dircon.co.uk   click on the ALE Controller tab.

[ *****  Day's notes refer to the original version of the program *****]

The program operates via the PC sound card.  There are two files, the

first which instals the program, the second which is COPIED into the

folder containing the installed files.  I find that on starting the

program (in W95) an error box appears advising "Cannot load

MScomm32.OCX". This lack does not seem to have affected the receive

functioning of the program.

As it was written as a personal project and not intended for general

distribution there are no help files or documentation; and no implied

support.

[ **** I shall try to have the file ALE.HTM made available via the

WUN Website. It includes notes by the author about the ALE

Controller program. View with Internet Explorer or Netscape  **** ]

One can run the program in two receive modes.

Single channel [Ch.00] using only the received audio from the receiver.

Channel scanning. This requires the addition of an RS232/V24 link between

one of the comports of the PC and the remote control port of the

receiver. This will allow the ALE controller to manage the receiver and

scan the designated channels.  A limited number of receivers are catered

for.

To date I've left all the CONFIGURATIONS/OPTIONS values as default.

In the Enable/Disable section I've set Display sound/Listen calls/Wild.

Tune in the channel where one has heard ALE activity.  Eg 18003/usb

(generally a whole, occasionally a half, kHz point).  Click the

button with the black triangle (Resume scan) and wait for the next ALE

activity to happen.  Note that addresses are often limited to three

characters.

Finally my thanks to the program's author, Charles Brain,  for the

following additional tips:

"You have the trace mode enabled. This was designed for a low level HEX

trace of the protocol and not for general operation. The [E] means the

remote ALE stopped sending, and the [R} means the controller was unable

to match it to any of its own callsigns and has resumed scanning. I put

it in there for my own bug fixing.

You want to enable display sounds and listen calls.

Most of the other options are needed only when you using it as an ALE

controller.

Items like All Calls determine whether the system will respond to an All

Call when scanning. DCD threshold sets the threshold when scanning; too

low and it will pause on every channel, too large and it will miss calls.

In its start directory you will find a file called owlog.txt.  That is a

text file which contains all the received callsigns. Also if you go into

the Other address menus you can see who you have worked."

With some systems disappearing or becoming less evident here's something

new for investigation.

-|-

############################   --o-o-O-o-o--  ##########################

You can get the program at:

http://www.chbrain.dircon.co.uk  The ALE Controller Program.

click on the ALE Controller tab.

Mac users can get Black Cat Systems' 'MultiMode' program which will

decode ALE (and many other modes) from:

http://www.blackcatsystems.com/software/multimode.html

For more information on ALE, try:

http://www.ncs.gov/n2/content/standard/files/f10473r3.pdf

(You'll need Adobe's Acrobat reader (Freeware))

http://www.cnn.com/TECH/computing/9909/20/radio.email.idg/

(Air Force tunes in radios for EMail link)

http://home.istar.ca/~racalcan/appnotes/ale.htm

http://webhome.globalserve.net/rlacroix/modems/ale.html

(The latter site has sound samples of ALE in Real Audio format).

Leif Dehio in Wiesbaden, a WUN member, has made available all sorts of

sound samples, including ALE, on his Web site at:

http://rover.wiesbaden.netsurf.de/~signals/WAV/MIL-STD-188-141A.WAV

The following are samples of modems used in conjunction with ALE:

http://rover.wiesbaden.netsurf.de/~signals/WAV/SWE-PSK.WAV

http://rover.wiesbaden.netsurf.de/~signals/WAV/MIL188-110ASHORT.WAV

http://rover.wiesbaden.netsurf.de/~signals/WAV/8021_VOC.WAV

Also see WUN's 'Digital Signals FAQ' Sections 1B and 1D, and the Sounds

section of the WUN CD.

There is IRC chat on the #flacom channel dedicated to ALE listening.

-|-

############################   --o-o-O-o-o--  ##########################

Setting Up & Running the ALE Controller Program.

-----------------------------------------------

Connect the receiver's audio output to either the Line-In or Mic-In on

the soundcard, and set their software levels by double clicking on

the volume icon in the system tray. You should hear the rig's audio

in the computer speakers.

Run the file ALE.EXE.

Use the Configurations|Options pull-down-menu (PDM) to get to the basic

settings. In the Enable/Disable panel tick 'All Calls', 'Any Calls',

'Display Sounds', 'Listen Calls', 'Wild', and 'Trace'. If you want to see

the low-level command strings tick 'Command Trace' as well. If you want

to see any operator chatter (opchat) which may accompany the link

setup, then tick both 'Trace' and 'Command Trace' - it produces a

rather untidy screen, but the value of the opchat as an aid to

identification may well make it worthwhile.

Clear any other ticks and 'OK' out to the main screen. If you intend to

use the program as a scanner with a suitable receiver, then tick the

Freq display' box also, otherwise leave it un-ticked. You will also have

to select your radio in the 'Radio Type' box and select the correct

serial (COM) port in the parameters panel of the Options screen.

Returning to the main screen, set the receiver to a quiet frequency and

adjust the gain levels such that the peaks in the spectrum display in

the 'Tune' panel on the main screen can only just be seen. The 'Volume'

panel does not work in my Windows '95 installation - The slider cannot

be moved. It only seems to work when scanning with the NRD545.

Find a frequency with ALE activity. In the European evening the easiest

ones to try are 2250, 4841 and 5120 kHz. 4841 and 5120 may be audible in

the Americas in their evening as well. These frequencies are used by the

Danish military for short-term comms and use real callsigns as

identifiers, mostly in the 'OW' series. Any appropriate USAF frequency

can usually be relied upon to produce copious ALE activity.

ALE activity is unmistakable. As Day Watson says, it's watery. A bubbling

interrupted sound usually in bursts with short intervals between them

which can go on for quite a few seconds sometimes. The audio files from

the sites listed above will help with identification of the ALE noises.

These can be used to test the soundcard and ALE Controller program setup

if you record them and then play them back into the soundcard. You can't

play them directly as the computer sound system will be needed for the

ALE program!

As an aid to tuning, the 'Tune' Panel's spectrum display will show

all 8 tones of the ALE signal as vertical bars. These are centred

between the black vertical guide bars when the signal is correctly

tuned. I know the signal is often all too brief, but very generally

the frequency will be on a whole kiloHertz or half (.5) kHz point,

mostly USB but sometimes on LSB. If the receiver readout gives a

unlikely frequency, while other indications are that the signal is

correctly tuned; if the signal produces no output, consider that you

may be listening to the wrong sideband.

If you are unable to scan using one of the supported scanning receivers

then leave the receiver tuned to a frequency where ALE activity is to

be found. Any ALE activity on the frequency will be processed by the

program. It is not necessary to use the scan function to monitor and

detect single-frequency activity.

It will occasionally happen that good quality ALE signals will not be

processed. This is most likely due to the information we are looking

for being masked or encrypted. (See the description of the Application

Level (AL) box below). This is a rather rare occurrence.

When an ALE signal is detected the information box at the bottom left

of the window will change from 'STOPPED' to 'SYNCHED' and when the ALE

burst is finished, the processed information will appear in the bottom

pane of the program screen, after which the info box will revert to

'STOPPED'. ('STOPPED' refers to the scanning function, it does not

mean that the program has stopped listening!)

Interpreting What You Receive.

-----------------------------

(The following OWLOG.TXT file entries are squashed horizontally to fit

the page. They are otherwise quite genuine intercepts from 3/4 May 2000).

('Trace' ticked)

[11:37:09][FRQ 17487000][SND][HHS     ][TIS][          ][AL0] BER 13 SN 03

[RX][CH 18][TIS HHS][TIS HHS][TIS HHS][R]

[11:39:15][FRQ 27870000][SND][HAW     ][TWS][          ][AL0] BER 21 SN 07

[RX][CH 25][TWS HAW][TWS HAW][R]

('Trace' off)

[12:16:12][FRQ 10238000][TO ][CH11    ][TIS][KAI1      ][AL0] BER 28 SN 19

Each of the items of the program's output is enclosed in square

brackets. (Boxes).

-----------------------------------------

[11:37:09]             The time the ALE activity was detected, taken

from your computer's time, but note: If the time

Time Box               is one hour ahead of UTC, then there is an

environment variable somewhere which is causing

the program to add an hour. Usually this is caused

by the SET TZ environment setting in AUTOEXEC.BAT.

REM it out or delete it.

-----------------------------------------

[FRQ 1748700]

Frequency              If you are scanning and have ticked the 'Freq

Box                    display' box in Config|Options, this box will

indicate the frequency on which the activity

was detected rather than the channel number (CH)

(in Hz).

WARNING: if you are unable to scan, then please

untick the 'Freq display' box, otherwise the boxes

probably won't display properly.

If you are scanning and have not ticked the

'Freq display' box in Config|Options, then the

Channel Number only will be displayed. (Channel

Numbers are assigned to successive frequencies

in a scan file commencing with CH 00). To convert

the channel number to frequency you will have to

look it up in Channels|List.

------------------------------------------

[RX][CH 18][TIS HHS][TIS HHS][TIS HHS][R]

Trace Derived Info     If 'Trace' is ticked, then this information will

Box                    be output, otherwise it is suppressed. It is

probably here that any 'opchat' will be seen.

It includes the channel number.

Can contain {TIS   ] and [TWS    ]

(Significance of [R} is not unknown).

Note: I cannot say much more about the Trace and

Command Trace facilities as I don't fully

understand them. Perhaps someone else....?

See also the note on Trace under 'additional

tips', by the program's author earlier on in

this discussion.

------------------------------------------

[CHN 18]

Channel No             The channel number. In single frequency

Box                    operation this will be 00.

------------------------------------------

[SND][HHS            ]

Type of                Can be [SND    ]  or  [TO           ]

Transmission/Address   SND = the station is 'sounding', i.e. announcing

Boxes                  its presence and transmitting a signal for link

quality assessment by other stations in the net.

TO = the designation of the intended recipient

of the ALE burst which will appear in the next

box.

[TO ][CH11           ][TIS][KAI1           ]

With [SOUND   ] the ident of the SENDING station.

With [TO      ] the ident of the individual

station to which the ALE activity is being

addressed.

Here KAI1 is sending to CH11.

TIS/TWS Box:           [TIS] = 'tis - i.e. 'it is', 'this is'.

The identity of the sending station, which will

be found in the next box. This is blank on

sounding transmissions, when the sender is found

in the address box.

[TWS] = 'twas - i.e 'this was'.

This seems to be a sort of termination

signal, present with sounding transmissions and

seen on termination of Danish Military data

transmissions.

------------------------------------------

[AL0] BER 28 SN 19

AL Box:                Application Level indicator.

Some users obviously need to protect their ALE

setup information from prying (WUN!) eyes, so

the setup information is encrypted.

AL0 = No Encryption.

AL1 = Unclassified, 24-bit lattice encryption with

a 1-minute protection interval.

AL2 = Unclassified.

AL3 = Unclassified but sensitive.

AL4 = Classified

Each ALE word is encrypted by XORing it with an

enciphered seed. The seed is a function of the

time of day, the frequency and an incrementing

counter.

Not much to be exploited here, and you'll only

ever see AL0, but it probably explains those

very occasional good quality ALE bursts  which

just won't synch - they're encrypted.

BER Box:               This is your monitoring system's ratings of the

signals it has recorded. SN being a signal-to

noise ratio. The higher the value the better the

signal.

------------------------------------------

The OWLOG.TXT File.

------------------

All ALE activity monitored and shown on the screen is saved in this file,

so that even if you clear the receive screen (first icon on the ALE taskbar)

the data isn't lost. It's an ASCII file, so it can be read with a text editor

such as Notepad and printed for later analysis if required. The system

doesn't start a new file with each listening session, new data is appended

to the previous data along with the date. Be aware of that extra hour

that may possibly be added to the times by the program.

-|-

############################   --o-o-O-o-o--  ##########################

Scanning.

--------

The program is designed to scan a list of up to 100 frequencies with

a suitable receiver and control system.

The receivers now supported directly by the program are:

EK890 FRG100, FT1000MP, FT757GX, FT847, GENERIC, HF1000, IC706,

IC707, IC726, IC735, IC746, NRD535, NRD545, IC-R75, IC-R8500, RX320,

SKANTI-825, TS850, TT550 and WJ8711.

Indirectly, additional receivers are supported where their serial control

parameters and internal microprocessor systems correspond to a receiver

type supported by the program. For example, the IC735 and IC736 have

identical serial control mechanisms so that the IC736 can be used with

the IC735 settings; the IC-R75 can be used with the IC706 selected.

(Although there would be no point, as the R75 is catered for).

No additional connections are required to scan other than your usual

rig-control interface e.g. CI-V for Icom. A squelch detect line is not

necessary as the ALE program doesn't rely on squelch: it will hold up

the scan when it detects a genuine ALE signal, process it and then

restart the scan.

As an aside: if you don't have the (expensive) CI-V for Icom, I have three

economy DIY designs which will do the same job.

Program Settings

----------------

In PDM Configuration|Options, select the appropriate radio in 'Radio Type'

(top right) and the appropriate 'Radio Com Port' in the Parameters panel

at bottom right. There may be scope for playing with the 'RX Audio Level'

setting, but I haven't tried this. Mine's set to 50. If you find that

the program lingers too long on a frequency after detecting and

processing ALE, adjust the 'Listen Time' box. Mine is set to 10.

Making a Scan File

------------------

Use the PDM Channels|Add:

o In the dialog box, in 'RX/Frequency' enter the frequency in Hz.

(Eight digits for frequencies above 10 MHz, seven digits for

frequencies below 10 MHz). i.e. 10581 kHz is entered as 10581000,

2250 Khz is entered as 2250000.

o 'Mode': enter either USB or LSB. Most ALE seems to be on USB.

o 'Usage': doesn't seem to matter - I've set them all to DATA.

o 'Permission': set to RX.

You don't need to enter a TX frequency.

Apart from the 'Groups' tick box, which I'll deal with later, ignore

all the other boxes.

The PDM 'Channels' allows you to Add, Delete, Modify and List channels,

all of which are self-explanatory. 'Goto' will set the receiver to the

selected channel.

Recorder Options.

----------------

Version 1.03j introduced an enhancement to permit SSB voice activity

following an ALE activation to be recorded as .WAV files. This facility

is set up in the Config�Recorder PDM. Tick Enable to enable the facility.

Set the maximum period of activity to be recorded, bearing in mind that

.WAV files take up a lot of disc space. I suggest 20 seconds maximum.

Then indicate the directory you wish the recorded .WAVs to be stored in.

There is no playback facility within the ALE Controller for these files,

so to play them back for analysis or recording you will have to use

another sound program such as Real Player, Cool Edit or Microsoft's

Sound Recorder in playback mode.

It may be necessary to tick the box 'Voice Detect' in the Configuration�

Options. Frankly I'm not quite sure. Try it anyway!

Organising Scan Files.

---------------------

100 frequencies is a lot to scan through if all the activity is in a

subset of those!

The ALE Controller program has a built-in means of grouping its channels,

which allows frequencies to be assigned to a particular group. This

will permit, for example separate lists of day and night frequencies

within the same scan file. (E.g. day frequencies are marked Group 'A' and

night ones Group 'B'.) A scan file could be split into, for example,

a list with the higher (day) frequencies designated Group A and

the lower (night) frequencies as Group B. Frequencies of an 'intermediate'

nature (appropriate to day and night groups) can both be ticked and will

be scanned in both groups. A single frequency can belong to up to eight

groups, A-H. When creating a scan file, indicate the group to which the

frequency belongs by ticking one or more of the 'Groups' boxes A to H in

Channels|Add/Delete/Modify. If you are modifying the scanfile with a text

editor, the group parameter is at column 37.

Note: you can only have a maximum of 100 frequencies in any one scan

file.

To scan a particular group: before starting the scanner, use the PDM

Configuration|Options and tick the group you wish to scan (one group

only allowed), OK out of the box and hit the scan button. This is the

5th button on the toolbar - a triangle. To pause (stop) the scan hit

the 4th button - the double bar. The first button - the white oblong

clears the receive screen. The processed data is still available,

however, in the file OWLOG.TXT.

You may find, with some older, slower receivers, particularly Icoms,

that if you mix USB and LSB channels, the receiver will be unable to go

back to USB immediately after scanning through an LSB channel. As

explained below by the program's author, this is a fault of the Icom

system. It isn't a problem with the IC706 and the IC-R75 which uses the

same control system.

It sometimes happens that pressing the 'Scan' button does not start

the receiver scanning. The info box at bottom left of the screen is

giving every indication that the scan has started, but the receiver

is not scanning. This will happen sometime when changes are made to

some items in Configurations�Options. If you are satisfied that the

receiver control hardware is correctly set up, close the ALE Controller

program and restart it. The 'Scan' button will now work. If not, then

you will have to check the hardware installation and check also that the

receiver's  serial speed and address are correct.  Check also that all

the frequencies in the scanfile are not all set to a group other than

that which you have selected to scan. I.e all the frequencies in the

scanfile are set to group A and you have selected to scan group B

in Configuration�Options.

There are three scan rates, 1, 2 and 5, which can be selected from the

PDM Configuration|Options. 5 is fastest. I seem to catch more activity with

it set to 2 rather than 5. The 5 rate is a bit too fast as the receiver's

AGC recovery time plus the latency in the Windows operating system

means the receiver is taking too long to recover its sensitivity and so

bursts are often missed. However, with modern receivers this will not be

a problem. Charles explained this and pointed out that he has not yet

perfected the scan stop routine. The ALE word takes 392 mS to send, so

you will notice that some good bursts get lost. Having said that, he

may well now have incorporated these enhancements to v1.03j.

Note: changing the scan group or scan speed in Configuration�Options does

not require a program restart.

Inputing a long scan file via Channels|Add can be a fairly long-winded

process.   After having built a scan file by this method, you can

export it via the PDM Fill|Dump. This will save the scan file information

to disc, via a standard file dialogue box, as an ASCII file with whatever

name you may wish to give it, in a directory of your choice. It will

have a '.QRG extension by default).

With care you can add, modify, delete and rearrange the frequency

information stored in a .QRG file. Use a text editor such as Notepad

to make changes, but be very careful to change only the frequency and

scan group information. To add a frequency I usually copy a line and then

overtype it with the frequency to be added. Watch the shorter line

lengths for frequencies below 10 MHz. With a little imagination, you

can construct completely new scan files from scratch this way. But

no more than 100 frequencies, please.

When you've done your modifications, save the file and go back into

the ALE program. If you now select PDM Fill|Load, the modified scan

file can be loaded back in from disc and used by the scanner.

If you get a 'SYNTAX ERROR' during the load process, this probably

means no more than a surplus line feed at the very end of the scan file.

This is harmless and you can use the file without any problems.

This process can be used to generate a selection of scan files. You

could have, for example, USAF, DIPLO and UNIDS scan files, all

different and separate from each other, loaded as and when required.

Long scanfiles can be split up into ranges such as DIP5-9.QRG;

DIP16UP.QRG etc.

Note: DUMP.TXT files from earlier versions of the program can be used

simply by changing their extensions to .QRG.

Load in scanfiles via Fill�Load which offers a standard Windows

file dialogue. To save a scanfile modified with Channels�Add,

Channels�Delete or Channels�Modify, use the PDM Fill�Dump.

To get Fill to look for scanfiles in the directory of your choice

rather than a directory of Windows' choosing, right click on the ALE

icon on your desktop, select Properties and enter the directory of your

choice in the 'Start In' box, then hit OK. You do of course have this icon

on your desktop??!

Receiver Compatibilities & Parameters.

-------------------------------------

Your Receiver:    Select        Serial     Machine

Radio         Port       Hex

Type:         Speed      Address

-------------     ------        ------     -------

IC736             IC735         1200         04  Set on Rx

IC-R75            R75           19200 (HI)   5A  Set on Rx

IC-R8500          R8500         19200 ?      4A  Set on Rx ?

TS870             TS850         4800         ??

NRD545            NRD545        4800         --  Automatic

Perhaps others can add to this list.

These notes, from Alan Doherty, may help those who do not have a

supported or compatible receiver:

"Hi All,

If your receiver is not yet supported in ALE V1.03d, and you want to

try scanning several ALE channels, read on. I tried a little messing

around with separate scan programs. First I loaded several ALE

frequencies into Scancat for Windows, and set it up to stop on a

station, had about 70% success on the stronger signals. Then I

programmed my Kenwood TS-870 memories with the same channels, the

success rate was not as good this time as the squelch had to be set

quite high to enable the scan. One problem with this method, you won't

get a log of the active frequencies, but all in all it still proved to

be better than sitting for hours and hours monitoring a single channel !

This setup should work for any computer controlled receiver and

compatible software.

BTW, for any TS-870 users if you set the baud rate to 4800 and choose

the TS-850 setting, it works more or less as it should do from within

the ALE software, (though the mode changeover does not work).

Have fun...73 de Alan Doherty, GI0OTC"

############################   --o-o-O-o-o--  ##########################

>From the author of the program:

Reply-To: Charles Brain

Hello Folks,

Frequencies are entered in Hz

You can add up to 100 frequencies then put them into different scan groups

so you can scan different groups at different times. Membership of a scan

group is done from the channel dialog box and the current scan group is

selected from the options dialog.

When adding a freq you only need to input the RX frequency.

The software was not originally written for the SWL community hence the

reason for channels rather than frequencies in the display, to a non

technical user frequencies mean nothing.

[ *****  This has been fixed from V1.03d on. Frequency display is now *****]

[ *****  an available option. Thanks Charles.                         *****]

The problem with the USB/LSB switching is due to older Icom radios not

being able to process more than one command at a time, if you send two

commands in quick succession only the second one will be processed. I added

timing/queing mechanism to get round this but obviously the timeout period

is too short, its fine on my IC706.  [ **** and on the R75 **** ]

The reason so few radios are supported is that I only have 3 different SW

radios here so I have to rely on other people to supply information / test

the software. Also the ALE S/W comes last in priority, well behind stuff

that makes me money, sorry.

I do listen to people's comments and will make changes when I am able to.

- Thanks

- Charles G4GUO

-|-

############################   --o-o-O-o-o--  ##########################

PCAle Controller - PC-ALE Project

---------------------------------

Introduction to PCALE  v 1.03 g

-------------------------------

A few years ago I started on the development of an ALE system for the P.C.

which started as a training exercise in DSP and grew from there.

The links below provide some light bedtime reading, for those insomniacs

amongst you.

ALE SOUND taken from NTIA test disk)

[http://www.chbrain.dircon.co.uk/ale.wav]

Final Version of MIL-STD 188-141B (1.5 M zipped )

ITS ALE Users Handbook (Offsite Link)

The controllers database can be saved to floppy disk in man readable form

and then distributed to all users of the net, this allows centralised net

management.

Since writing the above I have now restarted work on this because I am

using it to provide a backup orderwire for my digital voice experiments.

The software has become quite popular in the Utility listening community.

I am gradually adding new radios but this has to take second place to my

more commercial work.

I have recently entirely re-written all the signal processing modules and

those of you used to previous versions of the software hopefully will find

from version G onwards a significant improvement in performance. The software

is now totally free of any Intel DSP code.

If you would like to try this software it may be downloaded using the link

below. You will need at least a 133 Mhz Pentium and a soundcard. Also this

software is highly experimental so may not work on your machine. Limited

technical support will be provided via Email.

It is best to download both files, install the first file then copy the

second file to the sub directory that contains the executable.

If you have any problems I would appreciate knowing about them, so I can

correct them in a latter release.

There now exists an FAQ for this software which can be found at the WUN

site.

Software for Win95 Floppy Version (G) 1.8 MB

[http://www.chbrain.dircon.co.uk/pcaled.zip]

Latest executable Ver 1.03k (only for upgrading current version)

[http://www.chbrain.dircon.co.uk/ale.exe]

-|-

############################   --o-o-O-o-o--  ##########################

Additional Information.

----------------------

Here are some frequencies, with users where known. Start with the USAF

frequencies as the USAF is a prolific ALE user. In Europe, look at 2250,

4841 and 5120 kHz (Danish Military) initially to get a feel for the system.

It's also useful to build up some sort of database or list of ALE

frequencies from the logs submitted to the WUN server by stalwarts such

as Jack Metcalf, Sandy Abel (USAF expert) and Day Watson. I will be happy

to provide such files as I have available by EMail to whoever may require

them. Please ask.

Bear in mind that ALE itself is only a setup system for some other

system, which may be either data or voice. So what you are looking at

is often merely the setup, not the link itself. The link it sets up will

not necessarily turn up on the frequency you get the ALE data on, although

sometimes it does. The Danish Military for example often send data bursts

on 2250/4841/5120 kHz following ALE activity.

WUN member Mike Chace maintains a useful database of frequencies and

ALE identifiers. Mike sees many ALE loggings marked 'UNID' which can be

readily identified from the ALE-IDs database located at his Utility

Monitoring Central site at:

http://www.mindspring.com/~mike.chace/identa.html

Freq:   User:                     Freq:   User:

----    ----                      ----    ----

2250   Danish Mil                10190   ALGERIAN

4721   USAF                      10581   Swedish Diplo

4841   Danish Mil                11073.5 USAF

5120   Danish Mil                13215   USAF

5784   Unid Diplo                13375   UNID

6481   Unid Diplo                13390   Unid Diplo

6689   Unid Diplo                15016   USAF

6715   USAF                      15043   USAF

6721   USAF                      15860   Swed Diplo

6863   Romanian Diplo            16105   Swed Diplo

6889   Romanian Diplo            16105   USAF

6980   Swedish Diplo             18003   USAF

6981   Unid Diplo                19060   UNID

7375   ALGERIAN                  19340   Austrian UN

7375   USAF                      20031   USAF

7672   USAF                      20958   Swed Diplo

7972   Swedish Diplo             23337   USAF

8965   USAF                      24268   Unid Diplo

27870   USAF

All USB.

(Some of these may be inactive now : keep an eye on the WUN logs for

up-to-date loggings).

-�-

############################   --o-o-O-o-o--  ##########################

Addresses

---------

The addresses used in ALE activity can vary from the obvious to

the obscure.

USAF ground stations are usually identified by the last three letters

of the station's ICAO location indicator; Swedish diplomatic entities

are identified by 'S' followed by two figures, amost all of which are

common knowledge to WUN-ers. Others such as Chinese diplomats are identified

by such as 'Z1234' which tells us little. On the other hand, Danish military

stations often use callsigns from the International series, such as OWD,

many of which have been identified.

USAF aircraft using ALE employ a string of numbers as ALE identifiers

which can easily be converted to tail numbers and thence to aircraft

type. If you need the conversion data, please EMail me.

I also have a database of addresses. You can't have the database, but

you can EMail me for possible identification of an address. Or ask via

the WUN Listserver - someone may know.

Finally a plea. Please submit your logs in the WUN format if you wish

them to go into the monthly Newsletter. This example shows how you

should do it:

Raw data from the screen or OWLOG.TXT:

[16:42:13][FRQ 12101000][TO      ][S00            ][TIS][S51            ]

is logged in the form Freq Call: ID Time Mode Traffic/Remarks Date Sig

as something like:

12101   S51: Swedish Embassy Teheran 1642 ALE/USB Calls S00/MFA Stockholm

(25/02/01) (ABC)

or:

[19:01:05][FRQ  7966000][SOUND   ][5556           ][TIS][               ]

as, for example:

07966   5556: Unid 1901 ALE/LSB Sounds. (15/Feb/01) (AW)

-|-

############################   --o-o-O-o-o--  ##########################

USAF Users' Addresses:

---------------------

(These listings are from the DOD Flight Information Handbook.)

NW1/4 = Nightwatch 1/4

AF8 = Prob SAM Aircraft          OFF = Offutt AFB NE USA

AED = Elmendorf AFB AK USA

ADW = Andrews AFB MD USA         PLA = Lajes Field AZR

CBW =

DDF = Frankfurt-Main AF GER

DOV = Dover AFB DE USA

CRO = Croughton AB G             RIC = CAP Richmond VA USA

GTL = Thule AB  GRL

RSC = Rockwell Service Centre Cedar

GUA = Anderson AB  GUM                 Rapids, IA USA

HAW = Ascension Island ASC

HIK = Hickam AFB HI USA          TAG = Incirlik AB TUR

ICZ = Sigonella AB I

JDG = NAS Diego Garcia DGA

JNR = Salinas/Roosevelt

Roads PTR

JTY = Yakota AB J

LOU = Tinker AFB OK USA or

Bowman Fld Louisville

KY USA                     WRL = Warner Robins AFB GA USA

MCC = West Coast CA USA          Numbers are usually tail-numbers of

MDC =                              aircraft.

(Thanks to Jack Metcalf for these).

---------------------------------------------------------

Swedish Diplo

-------------

S00 = MFA Stockholm - Callsign SAM

Other S.. numbers equate to embassies listed under the callsign SAM

followed by the appropriate two-figure suffix.

---------------------------------------------------------

-|-

############################   --o-o-O-o-o--  ##########################

An Interpretation Exercise

--------------------------

This came via my IC736 ALE scan program while I was writing a previous

version of these notes. The activity record comes from the OWLOG.TXT

file.

I've cut off rightmost [AL] and [BER] data to keep the line length

reasonable - it's irrelevant to this discussion anyway.

1....Some Swedish Diplo:

[20:07:17][FRQ  6980000][TO      ][S17            ][TIS][S00            ]

[20:10:06][FRQ  7972000][TO      ][S17            ][TIS][S00            ]

At 2007 UTC, MFA Stockholm [S00] calls its embassy in Kiev, Ukraine, [S17]

twice. Once on 6980 kHz and three minutes later on 7972 kHz.

-------------

[16:42:13][FRQ 12101000][TO      ][S00            ][TIS][S51            ]

[16:45:13][FRQ 18686000][TO      ][S51            ][TIS][S00            ]

In this item the Swedish embassy in Teheran, Iran [S51] calls MFA Stockholm

on 12101 kHz at 1642UTC. MFA Stockholm returns the call three minutes later

on 18686 kHz.

---------------------------------------------

2....Sounding.

[19:01:05][FRQ  7966000][SOUND   ][5556           ][TIS][               ]

At 1901 UTC the unidentified Algerian(?) station 5556 makes its presence

known on 7966 kHz.

---------------------------------------------

3....Odds & Ends.

[19:48:51][FRQ  8130000][TO      ][KARIM3         ][TIS][O13            ]

At 1948 on the same day, an unidentified outstation [O13] calls the

Ministry of Information [KARIM3] in Algiers, on 8130 kHz. (The actual

identification of KARIM3 has yet to be confirmed).

[19:54:29][FRQ  8130000][TO      ][C13            ][TIS][O13            ]

Later, our unidentified friend [O13] calls another unknown outstation

[C13] on 8130 kHz.

[19:26:47][FRQ  7620000][TO      ][VKZ            ][TIS][VKF            ]

At 1926UTC on another day, unidentified user VKF calls another unidentified

user VKZ on 7620 kHz. These are suspected to be Australian military or

air force stations using their international callsigns.

---------------------------------------------

4....And Finally...

[14:10:06][FRQ 13370][SOUND   ][PLA            ][TWS][               ]

[14:10:14][FRQ 18003][SOUND   ][PLA            ][TWS][               ]

[14:11:00][FRQ 27870][SOUND   ][JNR            ][TWS][               ]

At 1410, PLA (Lajes Field, Azores (USAF)) sounds on 23370 and 18003 kHz,

and a minute later, on 27870 kHz, Salinas/Roosevelt Roads AFB, Puerto Rico

sounds for link quality assessment.